- Ministry of Education

- Sector: Construction

- Sub-sector: Water works construction

- Occupation: Water Well Drilling and Construction L-III

Materials and supplies (Consumable)

| No | Items | Qty | Unit | Specification |
| --- | --- | --- | --- | --- |
| Project 1: Performing well rehabilitation and pumping test |  |  |  |  |
| 1 | Fuel | 100 | Ltr. | Standard |
| 2 | Petrol | 2 | Ltr. | Standard |
| 3 | Foam | 10 | Ltr. | Standard |
| 4 | Well rehabilitation chemicals | 200 | gm | Chlorine/HCl/Sulfamic/hydroxylacetic |
| 5 | Oil |  |  |  |
| 5.1 | Engine oil | 1 | Ltr. | 15W40 |
| 5.2 | Compressor oil | 5 | Ltr. | Telux68/ Azolla68 |
| 5.3 | Hydraulic oil | 2 | Ltr. | Telax68/ |
| 5.4 | Gear oil | 1 | Ltr. | SAE90/140 |
| 6 | Thread grease | 1/2 | Kg. | Copper coated |
| 7 | Normal grease | 1 | Kg | Standard |

Tools & Equipment

| No. | Items | Qty | Unit | Specification |
| --- | --- | --- | --- | --- |
| Project 1: Performing well rehabilitation and pumping test |  |  |  |  |
| 1 | Rig/crain/tripod | 1 | Pcs | Minimum 4 ton lifting capacity |
| 2 | Compressor | 1 | Pcs | 12 bar |
| 3 | Bailer | 1 | Pcs | 4” |
| 4 | Surge block | 1 | Pcs | 55/8” |
| 5 | Socket | 1 | Pcs | Standard |
| 6 | Jar | 1 | Pcs | Standard |
| 7 | Stem | 1 | Pcs | Standard |
| 8 | Bit | 1 | Pcs | 4” regular bit |
| 9 | Generator | 1 | set | 25kVA |
| 10 | Submersible /Centrifugal pump | 1 | Pcs | 12KW |
| 11 | Switch box | 1 | Pcs | Standard |
| 12 | Pipe* | 1 | Pcs | GIP 2” |
| 13 | Gate valve | 1 | Pcs | GIP 2” |
| 14 | Elbow | 2 | Pcs | GIP 2” |
| 15 | Nipples | 2 | Pcs | GIP 2” |
| 16 | Pipe wrench | 2 | Pcs | 24”, 36” |
| 17 | Reducer | 2 | Pcs | GIP 2 1/2”—2” |
| 19 | V-notch | 1 | Pcs | 1mx1mx2m |
| 20 | Empty Barrel | 1 | Pcs | 200ltr |
| 21 | Stopwatch | 1 | Pcs | standard |
| 22 | Deep meter | 1 | Pcs | 200m. |
| 23 | Clamps | 2 | Pcs | GIP 2” |
| 24 | Bolt and Nut | 4 | Pcs | 22mm |
| 25 | Meter | 1 | Pcs | 5m long |
| 26 | Steel ruler | 1 | Pcs | 50cm long |

* Quantity will be determined as required